Ievgenii Gorbatyuk, Dariia Tverda

(Kyiv, Ukraine)

# efficiency of use in construction during earthwork of machines and mechanisms

Earthmoving equipment in highly developed countries occupies a leading place among self-propelled and trailed equipment for various purposes.

The basis of such technology is the scientific and technical principles of creating low-energy technologies and machines for the destruction of natural and artificial environments (soils, rocks, silts, reinforced concrete, bricks, etc.) in various conditions (ground - road, cultivation of agricultural land, engineering, military and emergency rescue operations, soil cleaning from pollution, reclamation, creation of trenches, channels, pits, trenches, repositories, etc.; underground - mining, construction of tunnels, etc.)

Earthwork is one of the most labor-intensive construction processes and therefore it is necessary to determine ways to improve the design of earthmoving machines through a reasonable feasibility study of various types of machine samples. Thus, the main objective of this work is to assess the effectiveness of earthmoving machines using the value of production potential as a criterion, which will allow a more reasonable feasibility comparison of different types of machine samples.

Probabilistic evaluation of the efficiency of earthmoving machines using the value of production potential as a criterion makes it possible to more reasonably compare machine samples and determine ways to improve their design. The value of the production potential can be used in forecasting annual output, estimating energy intensity, metal consumption, payback period, the economic effect of introducing a new sample or modernizing a earthmoving machine, as well as in selecting conditions and analyzing the results of production tests of earthmoving machines.

The mechanical method of excavation involves the sequential implementation of the following operations: separation from the natural massif (digging) of soil; transportation to the place of laying and unloading; treatment of the earth structure (leveling, compaction, etc.). Machines for earthworks are used in industrial and civil construction when planning sites, developing pits, trenches, in automobile and railway construction - when arranging excavations, embankments of the roadbed, etc. [1].

General-purpose earthmoving machines (excavators, bulldozers, scrapers, motor graders) are used in a wide range of soil conditions, and in the process of their use a relatively frequent change in construction facilities is characteristic. This allows us to conclude that the production of earthmoving machines is impersonal in relation to soil conditions, the change of which is random, and therefore the assessment of the technical and economic efficiency of earthmoving machines requires a probabilistic approach [2].

The average expected output of the machine and mechanisms can be estimated by its production potential, that is, by the mathematical expectation of its operational productivity:

П=KtПipi,	(1)

where Пipi – the mathematical expectation of technical productivity with the probability of operation of the machine in the soil of the i-th group of the difficulty of development; Kt – a coefficient that takes into account the use of the machine in time and is the product of the calculated coefficients of use of the machine in time in the year and during the shift.

The coefficient Kt depending on the size of the machine can vary widely and is, for example, for bulldozers and scrapers of traditional design 0.26-0.47 [3]. For earth-moving vehicles (bulldozers, scrapers), the production potential can be determined taking into account the range of soil movement in the form of:

ПL=KtLПipi,	(2)

where L – the range of soil movement corresponding to the conditions of the greatest expediency of using a machine of this size.

As an analogue of the value can serve, for example, the concept of ton-kilometer, used in the assessment of cargo transportation, but taking into account the intensity of the process. The product of the value of the production potential per annual fund of working time leads to the well-known concept of the production capacity of the machine [4].

In this case, the production potential characterizes the capabilities of the earthmoving machine, which can be realized during its operation in some of the most likely soil conditions and the fulfillment of the specified indicators of machine use in time.

Depending on the actual operating conditions, the average productivity of each individual machine may be higher or lower than its production potential. In these cases, the measure and nature of such a deviation will indicate the specifics of the soil conditions created, or assess the level of work organization. The concept of the production potential of an earthmoving machine allows in the most general form to determine its production capabilities. This approach also makes it possible to most objectively assess the effectiveness of various earthmoving machines, since it excludes accidents and subjective manifestations in the choice of initial soil conditions for technical and economic analysis.

Expression (1) shows that in order to determine the production potential of an earthmoving machine and its analysis, it is necessary to know the distribution of soils in the faces and the pattern of change in its technical productivity depending on the soil group.

When determining the technical productivity of the earthmoving machine as a function of the soil group, the influence of the parameters of the working bodies should be taken into account. For the digging process, this effect is determined by the resistivity of the soil dig K'. In general, it can be assumed that the change in value K' depending on the soil group occurs along some geometric progression with the denominator j. Then the specific resistance of the soil dig of the i-th group can be expressed as

K'i=ji−1K'1,	(3)

where K'1 –  specific resistance to soil digging; i – 1, 2,..., 5.

Taking into account the above and based on the analysis of energy costs for the main elements of the working cycle (digging, soil movement, idling), the technical productivity equation has the form [5]:

for earthmoving machines of cyclic action

Пi=190N(1−λL)1,3ji−1K'1+0,9(2−λL)L+0,4NL(10−i)q,	(4)

for continuous machines

Пi=270(Nηт−Nf)Kв.к1,2ji−1K'1,	(5)

where N – the power of the base machine; L – the range of movement of the soil; λ – a coefficient that takes into account the consumption of soil during movement; q – the capacity of the working body; ηт – the efficiency of the transmission; Nf – the power that is required to move the machine; Kв.к – a coefficient that takes into account the consumption of soil when digging.

These equations show that the production potential of the machine increases with a decrease in the value j that characterizes the rate of change in the specific resistance of the dig with a change in soil conditions. This indicates the feasibility of activating the working bodies of earthmoving machines. As you can see, the improvement of working bodies is one of the main ways to increase the production potential of earthmoving machines

Probabilistic evaluation of the efficiency of earthmoving machines using the value of production potential as a criterion makes it possible to more reasonably compare machine samples and determine ways to improve their design. The value of the production potential can be used in forecasting annual output, estimating energy intensity, metal consumption, payback period, the economic effect of introducing a new sample or modernizing a earthmoving machine, as well as in selecting conditions and analyzing the results of production tests of earthmoving machines.

References:

- Вольтерс О.Ю., Горбатюк Є.В., Русан І.В., Терентьєв О.О., Свідерський А.Т., Делембовський М.М., Куліков О.П. (2022). Базові машини будівельної техніки: підручник. Київ: ЦП Компринт. 494 с. Vol'ters O.Ju., Gorbatjuk Je.V., Rusan I.V., Terent'jev O.O., Sviders'kyj A.T., Delembovs'kyj M.M., Kulikov O.P. Bazovi mashyny budivel'noi' tehniky [Basic construction machinery]. Kyiv: CP Komprynt. [in Ukrainian].

- Pelevin L., Gorbatyuk Ie., Terentyev O., Sviderskyi A. Methodological and criterion bases of the study of the functioning of engineering complexes in the creation of target objects of the construction industry. In collective monograph “Technical research and development”. Іnternational Science Group. Boston: Primedia eLaunch, 2021. 396-401. https://doi.org/10.46299/ISG.2021.MONO.TECH.I.

- Горбатюк Є.В., Тереньєв О.О., Макарчук О.В. (2023). Методологічні і критеріальні основи дослідження функціонування інженерних комплексів при створенні цільових об’єктів будіндустрії. International scientific journal «Grail of Science» NGO European Scientific Platform (Vinnytsia, Ukraine) LLC International Centre Corporative Management (Vienna, Austria), 27. 258-264. https://doi.org/10.36074/grail-of-science.12.05.2023.040.

- Gorbatyuk Ie. V. (2024). Use of fuzzy models in engineering management. International scientific-practical conference “Modern science: problems, prospects, innovations”: conference proceedings (Aarhus, Denmark, December 21, 2024). Aarhus, Denmark: Scholarly Publisher ICSSH. 34-36.

- Горбатюк Євгеній. Ефективність впровадження у виробництво машин для земляних робіт. Проблеми та перспективи розвитку сучасної науки в країнах Європи та Азії: матеріали L Міжнар. наук.-практ. інтернет-конференції. Переяслав, 2022. С. 159-161.